CASE WESTERN RESERVE UNIVERSITY

Case School of Engineering

Department of Electrical Engineering and Computer Science

ENGR 210. Introduction to Circuits and Instruments (4)

Final Exam	12/15/03

Name: _________________________

Instructions

1.	Please write your name on the above line.

2.	You may use the front or back of any of the pages in the exam for intermediate work.

3.	Please be sure your final answers to each problem are written in the space provided.

4.	Notes and books are not permitted; any required formula sheets have been attached.

5.	Calculators are permitted.

6.	There are six problems, weighted as shown in the following table.

7.	You have 180 minutes to complete the exam.  If you finish early, you may quietly leave.  Have a nice holiday !

Score	Points

Problem 1		15

Problem 2		15

Problem 3		20

Problem 4		15

Problem 5		20

Problem 6	___	15

Total		100

Problem 1 (15 points)

- Write the node voltage equations for this circuit.
Hint: convert resistor values to conductance values, before proceeding.

| Node | Node-Voltage Equation |
| --- | --- |
| A | ____________ vA + ____________ vB = ____________ |
| B | ____________ vA + ____________ vB = ____________ |

- Use any method to find the proportionality factors for this circuit, and complete the following table.
Hint: the above matrix equation could be solved, but superposition, equivalent resistors, and current dividers is probably easier.

| Node | Node-Voltage Equation |
| --- | --- |
| A | vA = ____________ iS1 + ____________ iS2 |
| B | vB = ____________ iS1 + ____________ iS2 |

Problem 2 (15 points)
Sketch the following time-dependent waveforms.  Be sure to label the horizontal and vertical axes and/or points on the waveform to clearly indicate time and amplitude scales.

- v(t)=5−10e−100t V.  Note: e-1  0.368, e-2.3  0.1, e-4.6  0.01.

- i(t)=4+2cos(200π⋅t+π2) mA

- v(t)=3⋅u(t)−2u(t−1)+u(t−2) V

Problem 3 (20 points) 
Consider this time-dependent circuit in which the switch is initially open and no current is flowing.  The switch closes at time t = 0.

10

- Define and label the state variable in this circuit: 	______________

- Find the Thevenin equivalent of the circuit seen by the capacitor for t > 0.


VT = ______________


RT = ______________

- Write the initial value, final value, and time constant of the state variable. 


initial value = ______________


final value = ______________


TC = ______________

- Write the expression for the state variable for t > 0.


state variable = ____________________________

- Write the expression for iS(t) for t > 0.


iS(t) = ____________________________

Problem 4 (15 points)

- Consider a series RL circuit which has i(t) = 2 r(t) mA, where r(t) is the unit ramp function.  The inductor has value 4 mH and the resistor has value 10 k.  Calculate the following for time t = 3 sec.  Numerical answers are required.


resistor power = ______________


resistor voltage = ______________


inductor energy = ______________


inductor voltage = ______________


total charge flowed = ______________

- Consider a parallel RC circuit which has v(t)=5e−tTC V.  The capacitor has value 2 F and the resistor has value 5 k.  Calculate the following for time t = 4.6 TC.  Numerical answers are required.  Note: e-1  0.368, e-2.3  0.1, e-4.6  0.01.


time constant = ______________


capacitor charge = ______________


capacitor current = ______________


resistor power = ______________


capacitor energy = ______________

Problem 5 (20 points) 
This circuit is operating in the sinusoidal steady state.  The OPAMP is ideal.  Element values are 
	R1 = 4 k, C1 = 2.5 F, R2 = 5 k, C2 = 20 nF.

- Find the transfer function T(jω)≡V2(jω)V1(jω) using symbolic element values, i.e. ZR1, ZC1, ZR2, ZC2. 

T(j) = ______________________

- Find the impedance of each element, evaluated at the angular frequency  = 1000 rad/sec.

ZR1 = ___________ 	ZC1 = ___________ 	ZR2 = ___________ 	ZC2 = ___________

- Evaluate the transfer function obtained in part a) the impedances at  = 1000 rad/sec that were determined in part b).  Your final answer should be computed using magnitude/phase notation.


|T(=1000)| = ___________


T(=1000) = ___________

- Compute the phasors that represent the input voltage v1(t) and the output voltage v2(t) when v1(t) = 10 cos 1000t V, and write the result using magnitude/phase notation. 


|V1| = ___________


V1 = ___________


|V2| = ___________


V2 = ___________

Problem 6 (15 points)

- Find the specified voltage and currents given iS = 30 mA.

ix = ___________ 	iy = ___________ 	io = ___________ 	v0 = ___________

- Draw a block diagram that represents the function of this OPAMP circuit.  Your diagram should use two types of symbols: proportionality (a square) and summation (a circle).

vO

v1

v2

- Based on the block diagram draw in part b), write the value of vO in terms of v1 and v2.


vO = _______________________________